Program No. 1

Write a C program to perform Bit Stuffing

#include <stdio.h>

#include <string.h>

int main() {

char input[100], stuffed[200];

int i, j = 0, count = 0;

printf("Enter the bit stream: ");

scanf("%s", input);

for (i = 0; i < strlen(input); i++) {

stuffed[j++] = input[i];

if (input[i] == '1') {

count++;

if (count == 5) {

stuffed[j++] = '0';

count = 0;

}

} else {

count = 0;

}

}

stuffed[j] = '\0';

printf("Stuffed bit stream: %s\n", stuffed);

return 0;

}

Output:

Program No. 2

Write a C program to perform Character Stuffing

#include <stdio.h>

#include <string.h>

#define FLAG '$'

#define ESC  '#'

int main() {

char input[100], stuffed[200];

int i, j = 0;

printf("Enter the data string: ");

scanf("%s", input);

stuffed[j++] = FLAG;

for (i = 0; i < strlen(input); i++) {

if (input[i] == FLAG || input[i] == ESC) {

stuffed[j++] = ESC;

}

stuffed[j++] = input[i];

}

stuffed[j++] = FLAG;

stuffed[j] = '\0';

printf("Stuffed data: %s\n", stuffed);

return 0;

}

Output:

Program No. 3

Write a C program to perform Bit de-stuffing

#include <stdio.h>

#include <string.h>

int main() {

char stuffed[200], destuffed[200];

int i, j = 0, count = 0;

printf("Enter the stuffed bit stream: ");

scanf("%s", stuffed);

for (i = 0; i < strlen(stuffed); i++) {

destuffed[j++] = stuffed[i];

if (stuffed[i] == '1') {

count++;

if (count == 5) {

i++;

count = 0;

}

} else {

count = 0;

}

}

destuffed[j] = '\0';

printf("De-stuffed bit stream: %s\n", destuffed);

return 0;

}

Output:

Program No. 4

Write a C program to perform Character de-stuffing

#include <stdio.h>

#include <string.h>

#define FLAG '$'

#define ESC  '#'

int main() {

char stuffed[200], destuffed[200];

int i, j = 0;

printf("Enter the stuffed data: ");

scanf("%s", stuffed);

for (i = 1; i < strlen(stuffed) - 1; i++) {

if (stuffed[i] == ESC) {

i++;

}

destuffed[j++] = stuffed[i];

}

destuffed[j] = '\0';

printf("De-stuffed data: %s\n", destuffed);

return 0;

}

Output:

Program No. 5

Write a C program to implement CRC algorithm

#include <stdio.h>

#include <string.h>

void xorOperation(char *dividend, char *divisor, int pos) {

for (int i = 0; divisor[i] != '\0'; i++) {

dividend[pos + i] = (dividend[pos + i] == divisor[i]) ? '0' : '1';

}

}

int main() {

char data[100], divisor[50], dividend[150], remainder[50];

int dataLen, divisorLen;

printf("Enter the data bits: ");

scanf("%s", data);

printf("Enter the divisor (generator polynomial): ");

scanf("%s", divisor);

dataLen = strlen(data);

divisorLen = strlen(divisor);

strcpy(dividend, data);

for (int i = 0; i < divisorLen - 1; i++) {

dividend[dataLen + i] = '0';

}

dividend[dataLen + divisorLen - 1] = '\0';

for (int i = 0; i <= strlen(dividend) - divisorLen; i++) {

if (dividend[i] == '1') {

xorOperation(dividend, divisor, i);

}

}

strcpy(remainder, &dividend[strlen(dividend) - divisorLen + 1]);

char transmitted[150];

strcpy(transmitted, data);

strcat(transmitted, remainder);

printf("\nRemainder (CRC bits): %s", remainder);

printf("\nTransmitted Frame: %s\n", transmitted);

char recv[150];

printf("\nEnter received frame: ");

scanf("%s", recv);

for (int i = 0; i <= strlen(recv) - divisorLen; i++) {

if (recv[i] == '1') {

xorOperation(recv, divisor, i);

}

}

int error = 0;

for (int i = strlen(recv) - divisorLen + 1; i < strlen(recv); i++) {

if (recv[i] != '0') {

error = 1;

break;

}

}

if (error)

printf("Error detected in received frame!\n");

else

printf("No error detected. Frame is correct.\n");

return 0;

}

Output: